PROJECT REPORT

Arduino UNO Based Writing Machine

(CNC Pen Plotter)

A DIY CNC-based writing machine using stepper motors, servo motor,

GRBL firmware, Inkscape G-code generation and Universal G-code Sender

# 1. Introduction

A writing machine, also known as a pen plotter or CNC drawing machine, is an automated device that can draw, write text, or trace vector graphics on paper with high precision. This project presents a low-cost, DIY writing machine built using an Arduino UNO microcontroller, a CNC Shield, stepper motors for the X and Y axes, and a servo motor for the Z axis (pen up/down control).

The machine works by converting any SVG design or text created in Inkscape into G-code, which is then streamed to the Arduino via Universal G-code Sender (UGS). The Arduino runs the GRBL open-source CNC firmware, which interprets the G-code and drives the motors accordingly.

This project is ideal for students, hobbyists, and makers who want to explore CNC motion control, G-code programming, and embedded systems without high costs.

# 2. Objectives

- Design and build a low-cost 2D CNC writing/drawing machine

- Use Arduino UNO as the main microcontroller with GRBL firmware

- Control X and Y axes using stepper motors via CNC Shield

- Implement pen up/down control using a servo motor on the Z axis

- Generate G-code from vector designs using Inkscape

- Stream G-code to the machine using Universal G-code Sender

- Document the build process for replication and future improvements

# 3. Components Used

## 3.1 Hardware Components

| Component | Specification / Details | Quantity |
| --- | --- | --- |
| Arduino UNO | ATmega328P (Clone board used) | 1 |
| CNC Shield | V3 - Compatible with Arduino UNO | 1 |
| Stepper Motor Driver | A4988 or DRV8825 | 2 |
| Stepper Motor (X-axis) | NEMA 17 or 28BYJ-48 | 1 |
| Stepper Motor (Y-axis) | NEMA 17 or 28BYJ-48 | 1 |
| Servo Motor (Z-axis) | SG90 - for pen up/down | 1 |
| Power Supply | 12V DC, 2A or above | 1 |
| Pen / Marker | Standard ballpoint or marker | 1 |
| Frame / Chassis | DIY or 3D printed structure | 1 |
| Jumper Wires | Male-to-Male, Male-to-Female | As required |

## 3.2 Software Used

| Software | Version / Type | Purpose |
| --- | --- | --- |
| Arduino IDE | 2.x | Upload GRBL firmware to Arduino |
| GRBL Library | v1.1 | Open-source CNC motion control firmware |
| Inkscape | 1.x (Free & Open Source) | SVG design creation |
| Inkscape G-code Extension | gcodetools / J-Tech Plugin | Export SVG as .gcode file |
| Universal G-code Sender | Latest (UGS Platform) | Stream G-code to Arduino over serial |

# 4. System Design & Working Principle

## 4.1 Overall Architecture

The writing machine operates as a 2.5-axis CNC system. The X and Y axes provide planar (horizontal) movement for the pen, while the Z axis controls the pen height — whether the pen is touching the paper (pen down) or lifted away from it (pen up). The control flow is as follows:

- The user creates a design or writes text in Inkscape as an SVG file.

- The Inkscape G-code extension converts SVG paths into a .gcode file.

- The .gcode file is loaded into Universal G-code Sender (UGS).

- UGS sends G-code commands over USB serial to the Arduino UNO.

- GRBL firmware on the Arduino interprets the G-code and generates step/direction pulses.

- The CNC Shield routes these signals to A4988/DRV8825 stepper drivers.

- Stepper motors move the pen along X and Y axes. The servo controls pen height.

## 4.2 Z-Axis Servo Control

Unlike a traditional CNC that uses a stepper for Z-axis, this project uses a servo motor for pen up/down. GRBL controls the servo via its spindle PWM output on pin D11. The servo angle is mapped to pen-up and pen-down positions through GRBL's $30 and $31 parameters. The G-code commands M3 (spindle on = pen down) and M5 (spindle off = pen up) are used to control the pen.

# 5. Circuit Connections

## 5.1 CNC Shield Pin Mapping

| Signal / Component | CNC Shield Connection | Arduino Pin |
| --- | --- | --- |
| X-axis Stepper | X-axis Driver Slot | D2 (Step), D5 (Dir) |
| Y-axis Stepper | Y-axis Driver Slot | D3 (Step), D6 (Dir) |
| Servo Motor (Z) | Z+ Limit / Spindle PWM | D11 (PWM) |
| Enable (all drivers) | EN Pin on Shield | D8 |
| Power Input | 12V DC Jack on Shield | VIN |
| USB / Serial | Via Arduino USB port | D0/D1 (RX/TX) |

Note: Always power the CNC Shield from an external 12V supply. Do NOT power stepper motors from the Arduino's 5V pin as it cannot handle the current and may damage the board.

# 6. Firmware — GRBL Configuration

GRBL is an open-source, high-performance CNC firmware for Arduino. It accepts standard G-code over serial and outputs step/direction signals. Key GRBL settings for this project:

| GRBL Parameter | Value Used | Description |
| --- | --- | --- |
| $100 | 80 | X-axis steps per mm |
| $101 | 80 | Y-axis steps per mm |
| $110 | 500 | X max feed rate (mm/min) |
| $111 | 500 | Y max feed rate (mm/min) |
| $30 | 1000 | Max spindle RPM (pen down servo angle) |
| $31 | 0 | Min spindle RPM (pen up servo angle) |
| $32 | 1 | Laser/servo mode (enables M3/M5 servo control) |

Steps/mm values depend on motor type, microstepping setting, and belt/screw pitch. Calibrate by measuring actual movement vs commanded movement.

# 7. Software Workflow

## Step 1: Create Design in Inkscape

- Open Inkscape and create text or vector artwork

- Convert all objects to paths: Path > Object to Path

- Ensure all strokes are paths (not fills)

- Set document size to match your drawing area

## Step 2: Generate G-code

- Install G-code extension (Gcodetools or J-Tech Photonics Laser Tool)

- Go to Extensions > Generate from Path > G-code Tools

- Set Z-safe height, pen-up/down Z values, and feed rate

- Export the .gcode file to your computer

## Step 3: Send G-code via UGS

- Open Universal G-code Sender and connect at baud rate 115200

- Home the machine or set work origin ($H or G92 X0 Y0)

- Load the .gcode file in UGS

- Preview the toolpath and click Send

# 8. Challenges & Issues Faced

## 8.1 Clone Arduino UNO Issue

The project used a non-genuine (clone) Arduino UNO board. This resulted in only the Y-axis functioning correctly during initial testing. The likely cause is driver incompatibility or USB serial communication issues (CH340 chip vs genuine FTDI chip on original boards). Installing the correct CH340 USB driver resolves connectivity issues. Additionally, some clones may have minor voltage or timing differences that affect the CNC Shield's stepper driver enable signals.

## 8.2 Servo Z-Axis Calibration

Getting precise pen-up and pen-down positions required careful tuning of GRBL's spindle parameters ($30, $31) and the servo mount position. Too much pressure damaged paper, while too little caused incomplete lines.

## 8.3 Step Calibration

The steps/mm value needed calibration for accurate output. A commanded 100mm movement was measured and the $100/$101 values were adjusted proportionally until accurate.

# 9. Results

- The machine successfully moves along the Y axis and partially demonstrates motion control

- GRBL firmware was successfully flashed and communicates with Universal G-code Sender

- G-code was generated from Inkscape and loaded into UGS successfully

- Servo motor responds to M3/M5 commands for pen up/down

- The hardware assembly (frame, motors, shield) is complete and functional

- Full X-Y axis operation requires resolving the Arduino clone compatibility issue

# 10. Future Improvements

- Replace clone Arduino with genuine Arduino UNO or use a compatible board (like Arduino Nano with CNC shield adapter)

- Add a limit switch homing system for automatic machine zeroing

- Integrate a web interface or mobile app for wireless G-code sending

- Add an enclosure and cable management for a cleaner build

- Upgrade to NEMA 17 motors for better torque and precision

- Support for multiple pen colors via a tool changer attachment

- Implement automatic pen pressure control using a spring-loaded mount

# 11. Conclusion

This project successfully demonstrates the design and partial implementation of a low-cost Arduino-based CNC writing machine. The system architecture — combining GRBL firmware, stepper motors, a servo-controlled pen axis, Inkscape-based G-code generation, and Universal G-code Sender — forms a complete and well-integrated pipeline for automated drawing and writing.

While the use of a clone Arduino UNO introduced a hardware compatibility issue that limited full X-Y operation during testing, the core concepts, wiring, firmware configuration, and software workflow are all correctly implemented. With a genuine Arduino or proper driver installation, the machine is expected to operate fully.

This project is an excellent introduction to CNC systems, motion control, embedded firmware, and maker culture, and can serve as a foundation for more advanced plotters and drawing robots.

# 12. References

- GRBL Open Source CNC Firmware: https://github.com/grbl/grbl

- Universal G-code Sender: https://universalgcodesender.com/

- Inkscape Vector Editor: https://inkscape.org/

- CNC Shield V3 Documentation: https://blog.protoneer.co.nz/arduino-cnc-shield/

- A4988 Stepper Driver Datasheet: Pololu.com

- Arduino UNO Reference: https://docs.arduino.cc/hardware/uno-rev3/